PREINSTALL

This guide walks you through everything you need to do before running the main code on your Raspberry Pi. It covers required hardware components, wiring, OS configuration, and all software dependencies. Follow the steps in order, skipping any step is the most common cause of the game failing to start.

Materials:

Hardware Requirements:

| Component | Specification | Purpose |
| --- | --- | --- |
| Raspberry Pi 4 Model B | 2 GB or 4 GB RAM | Main processing unit — runs the game loop, sensor I/O, and signal processing |
| MPU6050 IMU Module | 6-DoF (GY-521 breakout) | Provides accelerometer + gyroscope tilt data via I2C |
| Female-to-Female Jumper Wires | 4 wires minimum | Connect MPU6050 pins to Raspberry Pi GPIO header |
| MicroSD Card | 16 GB+ Class 10 / A1 | Holds Raspberry Pi OS, Python, game files, and libraries |
| USB-C Power Adapter | 5V / 3A (official Pi PSU) | Stable power — underpowered supplies cause random crashes during gameplay |
| HDMI Monitor | Any resolution (micro-HDMI cable) | Display for the racing game — required since pygame needs a screen |
| USB Keyboard + Mouse | Any USB/wireless | Initial setup, terminal commands, and keyboard fallback for the game |
| Micro-HDMI to HDMI Cable | Pi 4 uses micro-HDMI | Connect Raspberry Pi to the monitor |

MPU6050 Wiring:

| MPU6050 Pin | Raspberry Pi Pin | GPIO Name | Notes |
| --- | --- | --- | --- |
| VCC | Pin 1 | 3.3V Power | 3.3V ONLY — never connect to Pin 2 (5V) |
| GND | Pin 6 | Ground | Any GND pin works (6, 9, 14, 20, 25, 30, 34, 39) |
| SDA | Pin 3 | GPIO 2 (SDA1) | I2C data line — must be this exact pin |
| SCL | Pin 5 | GPIO 3 (SCL1) | I2C clock line — must be this exact pin |

GPIO Pin Reference Diagram:

| Function | Pin Name | Pin # | Pin # | Pin Name | Function | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| VCC → MPU6050 | 3.3V PWR | 1 | 2 | 5V PWR | DO NOT USE | Connect MPU6050 VCC to Pin 1 (3.3V), never Pin 2 (5V) |
| SDA → MPU6050 | GPIO 2 (SDA) | 3 | 4 | 5V PWR | — | Connect MPU6050 SDA to Pin 3 |
| SCL → MPU6050 | GPIO 3 (SCL) | 5 | 6 | Ground | GND → MPU6050 | Connect MPU6050 GND to Pin 6 |

Enable I2C & Verify Sensor:

I2C is disabled by default on Raspberry Pi OS. The game cannot communicate with the MPU6050 until I2C is enabled and the Pi is rebooted.

- Enable I2C via raspi-config

- Open a terminal on the Raspberry Pi.

- Run the configuration tool:

$ sudo raspi-config

- Navigate to:  Interface Options  →  I2C  →  Enable  →  Yes  →  Finish

- Reboot the Pi when prompted, or manually run:

$ sudo reboot

- 3.2  Verify the MPU6050 is Detected

After rebooting, confirm the sensor appears at I2C address 0x68:

# Scan the I2C bus

$ sudo i2cdetect -y 1

Expected output: You should see  68  printed in the grid at row 60, column 8. If the entire grid shows dashes (--), recheck your wiring.

0  1  2  3  4  5  6  7  8  9  a  b  c  d  e  f

00:          -- -- -- -- -- -- -- -- -- -- -- -- --

10: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

20: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

30: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

40: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

50: -- -- -- -- -- -- -- -- -- -- -- -- -- -- -- --

60: -- -- -- -- -- -- -- -- 68 -- -- -- -- -- -- --

70: -- -- -- -- -- -- -- --

Software Installation:

There are two ways to do this.

Install all required system and Python packages in the order shown below. An active internet connection is required.

## Update the OS

Always update before installing packages to avoid version conflicts:

$ sudo apt-get update

$ sudo apt-get upgrade -y

## Install System Packages

These system-level packages provide I2C support and the tools needed to verify the sensor. Do not skip this step — smbus2 depends on the kernel I2C drivers installed here.

- $ sudo apt-get install python3-smbus i2c-tools python3-pip -y

| Package | Why it is needed |
| --- | --- |
| python3-smbus | Provides the kernel-level SMBus/I2C driver bindings for Python |
| i2c-tools | Provides i2cdetect and other utilities to verify the sensor is wired correctly |
| python3-pip | Python package manager — needed to install smbus2 and pygame |

## Install Python Libraries

Install the two required Python packages:

# Required: direct I2C communication with MPU6050

$ pip3 install smbus2

# Required: game rendering and input handling

$ pip3 install pygame

| Python Package | Version Tested | Purpose |
| --- | --- | --- |
| smbus2 | 0.4.3 | Communicates with the MPU6050 at register level over I2C — replaces the broken mpu6050 pip package entirely |
| pygame | 2.5.2 | Renders the racing game window, handles 60 FPS game loop, draws road/car/obstacles, and provides keyboard fallback controls |
| math | stdlib | Built into Python — provides atan2() for accelerometer angle calculation inside the complementary filter |

## Verify Installations

Confirm both libraries are correctly installed:

$ python3 -c "import smbus2; print('smbus2 OK')"

$ python3 -c "import pygame; print('pygame OK')"

Both commands should print OK. If either says  ModuleNotFoundError , re-run the pip3 install command from Step 4.3.

The other way is to do this on your raspberry pi command prompt.

- Type ls -a to show all of the projects

- Then type cd “name of file”

- After type ls- a again.

- Try doing pip3 install mpu6050-raspberrypi

- If it says “error: externally-managed environment”

- Do this :

python3 -m venv venv

source venv/bin/activate

pip install “name of the file”-raspberrypi pygame

- Then after installing try running:

pip freeze > requirements.txt

ls-a

- It should show a venv on the right side

- Run the code.

Testing the mpu6050 sensors:

- Copy the mpu6050_sensor_test

- Run it

- If you tilt the mpu6050 and it changes numbers your tilt sensor is ready.

Running the Game File:

- Copy the main code

- Run it

- If the display shows the game and you try to run it, you’re done and finished.